- Crystal Software

Configuring TextPipe for Unix faxes into GFI FAXmaker

- Application Integration

- Prepared By

- Crystal Software

| Last Update | : | 13-Dec-2000 |
| --- | --- | --- |
| Print Date | : | 27-Jun-2000 |
| Revision | : | 1.0 |
| Status | : | Final |
| Document | : | input.doc |

- Public domain

- Table of Contents

1	Introduction	1

2	Instructions	2

2.1	Download TextPipe Pro	2

2.2	Install TextPipe Pro	2

2.3	Determine the Unix incoming folder	2

2.4	Determine the GFI FAXMaker incoming folder	2

2.5	Configure the TextPipe filter	2

2.6	Automating TextPipe	5

2.6.1	Batch file to run TextPipe	5

2.6.2	Activating Windows NT’s Task Scheduler	6

2.6.2.1	Connecting to Network Drives	7

2.6.3	Defining a schedule	7

3	About TextPipe	8

4	Contact information	9

5	Resources	10

# Introduction

GFI FaxMaker allows you to send faxes that are placed in a predefined folder on your fax server. FaxMaker does not support Unix files, but with TextPipe Pro, you can convert your Unix-based faxes to a Windows format that GFI FaxMaker understands.

This document describes how to install and configure TextPipe Pro to do this.

# Instructions

## Download TextPipe Pro

The first step is to obtain a copy of TextPipe Pro. It can be downloaded from:

http://www.crystalsoftware.com.au

Please make sure you download the Pro version (the default) and not the Standard version, because the Standard version does not have command-line support.

## Install TextPipe Pro

Once TextPipe has downloaded successfully, allow the installation to proceed without any changes to the defaults. If you make any changes to the defaults you will have to adjust the instructions that follow accordingly.

## Determine the Unix incoming folder

You must define a folder where your Unix host will place the Unix-coded faxes (usually by FTP or File Transfer Protocol). This folder can be physically on your Unix host provided the fax server where TextPipe is installed can ‘see’ the files.

We will assume that the folder for these files is c:\incoming, but in practice, it can be any folder.

## Determine the GFI FAXMaker incoming folder

Next you must determine where converted files must be placed so that GFI FAXMaker can process them.

We will assume that this folder is C:\Program Files\GFI FAXmaker\incoming. The location of this folder depends on how FAXmaker is installed on your system. Please check your documentation to determine this.

## Configure the TextPipe filter

TextPipe needs to be configured to:

- process the Unix fax files from c:\incoming

- convert the Unix fax files to DOS fax files

- place the DOS fax file in c:\Program Files\GFI FAXmaker\incoming

Here is how this is achieved:

- Run TextPipe from the Start Menu

- Choose Open from the File Menu

- Open the filter list file GFI FAXmaker.fll, included with this document

- TextPipe then shows the loaded filter file (below):

Figure 1. TextPipe after loading GFI Faxmaker.fll (input filter displayed)

- Ensure that Delete input files after processing is checked on. This ensures that we won’t send any faxes twice. Note that the input filter is also configured to process binary files.

- Click on File Output: new base dir … in the tree view.

TextPipe changes to show the output filter:

Figure 2. TextPipe shows the output filter

- The box that says *** specify GFI Fax folder here *** is where we must tell TextPipe to put the converted DOS output files so that GFI FAXmaker can find them. Click on the folder box to the right of this field, and specify the folder: 

C:\Program Files\GFI FAXmaker\incoming

- Right. Almost there. Except we haven’t told TextPipe where the Unix files are coming from. Click on the Files to process tab.

TextPipe shows the files list:
 Figure 3. TextPipe shows the files to process tab

- This is where we need to specify where the input Unix files come from. Delete the highlighted text and type

c:\incoming

- Save your filter by clicking on Save in the File Menu.

- That’s it! TextPipe now knows what it needs to begin processing. You can test it out (once Unix files are in place) by clicking the Go button.

## Automating TextPipe

There are three elements to automating TextPipe:

- A batch file to run TextPipe with the correct command line parameters and with enough intelligence to not process Unix files while they are being sent from the Unix machine.

- Activating Windows NT’s Task Scheduler

- Defining a schedule.

### Batch file to run TextPipe

A batch file needs to be created with the necessary commands to run TextPipe. We recommend that this batch file be called C:\tprun.bat

The batch file should contain the following commands:

@echo off

rem Check for lock file which indicates Unix host is downloading files

rem and exit if it is

If exist c:\incoming\unix.lock goto exit

Rem Create a lock file so Unix knows we’re processing

Echo dummy text > c:\incoming\processing.lock

Rem Unix lock file does not exist – so we can process files

Rem Load TextPipe, run filter and then exit

"C:\Program Files\Crystal Software\textPipe.exe" /minimized "/f=C:\Program Files\Crystal Software\TextPipe\GFI FAXmaker.fll" /g /q

rem Remove lock file

del c:\incoming\processing.lock

:exit

The batch file contains commands that allow the Unix host to determine if TextPipe is currently processing files (and therefore the file transfer should be delayed). It also prevents TextPipe from processing any files if the file c:\incoming\unix.lock exists.

### Activating Windows NT’s Task Scheduler

The Windows NT scheduler can be used to trigger TextPipe on a predefined schedule e.g. every 5 minutes, every 10 minutes on weekdays etc.

First ensure the Task Scheduler service is started by going to Control Panel, Services and ensuring that Task Scheduler has Status="Started" and Startup="Automatic".

The Schedule service is initially configured to run in the System account on the local computer. When the Schedule service runs using this account, there are no restrictions on the jobs that can be executed by Schedule service. However, these jobs will have limited network access, because the System account on a local computer is not recognized by other computers.

To overcome network access limitations, you can configure the Schedule service to run in a user's account. Then, jobs executed by the Schedule service will be governed by network access of the user's account. However, since in this case Schedule service is not using the local System account, only jobs that do not require interaction with the user can be run successfully.

To configure the Schedule service to run in a user's account, use User Manager to give this user's account the "Log on as a service" right on a local computer, then in the Control Panel Services option, configure the Schedule service to startup using this user's account.

#### Connecting to Network Drives

Scheduled jobs that access the network should not use redirected drives made by the user. The Schedule service may not be able to access these drives and they may not be present if a different user is logged on at the time the scheduled job runs. Instead, scheduled jobs should use UNC paths. For example,

at 1:00pm my_backup \\server\share

and not

at 1:00pm my_backup x:

where x: is a connection made by the user.

If you schedule an at command that uses a drive letter to connect to a shared directory, you should include an at command to disconnect the drive when you are finished using it. If the drive is not disconnected, the drive letter assigned will not be available or seen at the command prompt.

### Defining a schedule

Use the Windows ‘at’ command to specify when a command should be run e.g.

at 04:00 /interactive /every:M,T,W,Th,F,S,Su "cmd.exe /c c:\tprun.bat"

You can use the provided batch files to automatically schedule TextPipe every 15 minutes (at schedule 15 minutes.bat). The batch file simply runs the ‘at’ command multiple times to define when the batch file  gets run e.g.

at 08:00 /interactive /every:M,T,W,Th,F "cmd.exe /c c:\tprun.bat"

at 08:15 /interactive /every:M,T,W,Th,F "cmd.exe /c c:\tprun.bat"

at 08:30 /interactive /every:M,T,W,Th,F "cmd.exe /c c:\tprun.bat"

at 08:45 /interactive /every:M,T,W,Th,F "cmd.exe /c c:\tprun.bat"

at 09:00 /interactive /every:M,T,W,Th,F "cmd.exe /c c:\tprun.bat"

# About TextPipe

TextPipe is a multi-file text transformation workbench supporting:

- Data warehousing - extract - transform - load (ETL processes)

- Electronic text processing - standardizing text to in-house formats by fixing common formatting errors and standardizing line breaks and paragraph formatting

- Web site publishing - update text patterns across huge web sites. Handle tagged data such as HTML, XML and SGML

- Format conversions - between DOS, Unix, Macintosh and Mainframe systems. TextPipe is a must- have tool in heterogeneous environments

- Data quality - investigate, cleanse and scrub data before import into a data warehouse or data mart, or after extraction for another purpose

- Programming - global file search/replace in large C, C++, Java, Visual Basic or Delphi projects

- Data mining - on competitor web sites. Copy their web site to your local machine using Offline Explorer, then extract the data using TextPipe (throw away advertising and formatting information)

- Data analysis activities - sort, remove duplicates or list duplicates...

TextPipe's power is delivered through the following key features:

- Search and replace

- Literal text (both single line, multi-line and binary character search)

- Text patterns, using egrep (perl-like) regular expressions

- Sounds-like matching (soundex)

- Multiple operations per file (eg apply multiple search and replaces at the same time, followed by Unix to DOS EOL conversion)

- Process any number of files and folders (with exclusion lists and subfolder depth settings), or the Windows clipboard

- No memory limit on the size of files processed (eg process 2GB file in 16MB memory)

- Test runs to verify results, as well as an ad-hoc area for testing conversions

- ActiveX scripting - write your own filters in JScript, VBScript, PerlScript etc

- TextPipe Engine can be embedded in your application to provide TextPipe's features to your application

- Interactive as well as non-interactive modes of operation (can be invoked from command line or scheduling tool). Results are automatically logged

# Contact information

| Email: | mail@crystalsoftware.com.au |
| --- | --- |
| WWW: | http://www.crystalsoftware.com.au |
| Fax: | +61-3 9800-3078 |
| Voice: | +61-3 9888-3104. After hours number. Please leave a contact name, number and time, and we will endeavor to get back to you within 24 hours. |
| Mail: | Simon Carter Crystal Software 2 / 9 Woodbine Court Wantirna Victoria 3152 AUSTRALIA. |

# Resources

TextPipe information is available from

http://www.crystalsoftware.com.au/textpipe.html

- TextPipe and other resources can be downloaded from

http://www.crystalsoftware.com.au/textpipe.exe